	SECTION 307-01: Automatic Transaxle/Transmission
	2000 Town Car Workshop Manual

	IN-VEHICLE REPAIR
	Procedure revision date: 01/29/2003




Digital Transmission Range (TR) Sensor

	Special Tool(s)

	[image: image1.png]



	Alignment Gauge, TR Sensor
307-351 (T97L-70010-A)


Digital Transmission Range (TR) Sensor
The digital transmission range (TR) sensor is located on the outside of the transmission at the manual lever. The digital sensor completes the start circuit in PARK and NEUTRAL, and the back-up lamp circuit in REVERSE. The digital sensor also opens/closes a set of four switches that are monitored by the PCM to determine the position of the manual lever (P, R, N, D, 2, 1).

Removal
1. Disconnect the battery ground cable. For additional information, refer to Section 414-01.

2. Raise the vehicle on a hoist. For additional information, refer to Section 100-02.

3. Disconnect the manual lever shift control cable.

[image: image2.png]AD1S25A




4. Disconnect the digital TR sensor connector.

[image: image3.png]



5. Remove the bolts and remove the digital TR sensor.

[image: image4.png]



Installation
1. Install the digital TR sensor and loosely install the bolts.

[image: image5.png]



2. NOTE: Manual shift lever shaft must be in the neutral position.

Using the special tool, align the digital TR sensor slots. The tool is designed to fit snugly.

[image: image6.png]



3. Tighten the bolts and remove the special tool.

[image: image7.png]10 Nm (62-89 Ib/in)

7

GD1346-A





4. With the manual lever in overdrive, connect the shift lever control cable.

[image: image8.png]



5. Connect the digital TR sensor electrical connector.

[image: image9.png]



6. Lower the vehicle.

7. Connect the battery ground cable.

